CE MARK  TECHNICAL FILE

AUSTRALIA EMC CONSTRUCTION FILE

of

Personal Computer

Model/ Type/ Machine Type

VT3300

Contains:

1. Declaration of Conformity

2. EN55022/CISPR 22, AS/NZS 3548 Class B  EMI test report

3. EN55024, AS/NZS 4252.1, EN61000-3-2, and EN61000-3-3 
test report

4. Certificate of EN60950

5. Block Diagram and Schematics

6. User`s manual

Date: 2001/08/01

ISL-01A063E

Acer Incorporated

7 Hsin Ann Road

Science-Based Industrial Park

Hsinchu 300, Taiwan

R.O.C.

Telephone: (03) 577-0707

Facsimile: (03) 577-8500

Declaration of Conformity

Name of Manufacturer:	Acer Inc.

Address of Manufacturer:	7 Hsin Ann Rd., Science-Based Industrial Park
	Hsinchu 30077
	Taiwan, R. O. C.

Declares that product:	Personal Computer

Model/ Type/ Machine Type:	VT3300

Assembled by:	Same as above

Address:	Same as above

Conforms to the EMC Directive 89/336/EEC as attested by conformity with the following harmonized standards:

EN55022:1994 /A1:1995 /A2: 1997: Limits and Methods of Measurement of  Radio Interference characteristics of Information Technology Equipment

EN55024: 1998: Information technology equipment--Immunity characteristics --Limits and methods of measurement

EN61000-3-2: 1995: A1:1998/A2:1998: Limits for harmonics current emissions

EN61000-3-3: 1995: Limits for voltage fluctuations and flicker in low-voltage supply systems

Conforms to the Low Voltage Directive 73/23/EEC as attested by conformity with the following harmonized standard:

EN60950:1992 /A1:1993, A2:1993 /A3:1995 /A4:1997 /A11:1997: Safety of Information Technology Equipment Including electrical business equipment

Conforms to the C-Tick Mark requirement as attested by conformity with the following standards:

AS/NZS 3548: 1995 /A1:1997 /A2:1997: Information technology equipment

AS/NZS 4252.1:1994: Generic Immunity

2001/08/09

---------------------------	------------------

Angus Hsieh / Sr. Director	Date

Acer Inc.	ISL-01A063E

EN55024 / AS/NZS 4252.1 / IMMUNITY

EN61000-3-2 / HARMONICS

EN61000-3-3 / VOLTAGE FLUCTUATIONS

TEST  REPORT

of

Personal Computer

Model/ Type/ Machine Type

VT3300

Applied by:

Acer Inc.

7 Hsin Ann Rd., Science-Based Industrial Park

Hsinchu 30077

Taiwan, R. O. C.

Test Performed by:

(NVLAP Lab. Code: 200234-0)

International Standards Laboratory

No. 21, Alley 37, Lane 122, Sec. 2
Hsiwan Rd. Hsichih

Taipei Hsien 22117
Taiwan, R.O.C.

Tel:(02)2646-2550

Fax:(02)2646-4641

Contents of Report

1.	General	1

1.1	Certification of Accuracy of Test Data	1

2.	Summary	2

2.2	Description of Equipment Under Test	2

2.3	Description of Equipment Under Test	8

3.	Electrostatic discharge (ESD) immunity	10

3.1	Electrostatic discharge (ESD) immunity test	10

4.	Radio-Frequency, Electromagnetic Field immunity	11

4.1	Radio-Frequency, Electromagnetic Field immunity test	11

5.	Electrical Fast transients/burst immunity	12

5.1	Electrical Fast transient/burst immunity test	12

5.2	Electrical Fast transient/burst immunity test for I/O cable	14

6.	Surge Immunity	15

6.1	Surge immunity test	15

7.	Immunity to Conductive Disturbance	16

7.1	Immunity to Conductive Disturbance	16

8.	Power Frequency Magnetic Field immunity	17

8.1	Power Frequency Magnetic field immunity test	17

9.	Voltage Dips, Short Interruption and Voltage Variation immunity	18

9.1	Voltage Dips, Short Interruption and Voltage Variation immunity test	18

10.	Harmonics	19

10.1	Harmonics test	19

11.	Voltage Fluctuations	22

11.1	Voltage Fluctuations test	22

12.	Test Equipment List	23

12.1	Software for Controlling Spectrum/Receiver and Calculating Test Data	24

13.	Photographs	25

13.1	Photos of ESD measurement	25

13.2	Photos of RF Field Strength Susceptibility Measurement	25

13.3	Photos of Electrical Fast Transient/Burst measurement	26

13.4	Photos of Surge measurement	26

13.5	Photo of Conductive Measurement	27

13.6	Photo of Magnetic field measurement	27

13.7	Photos of Voltage Dips measurement	28

13.8	Photos of Harmonics and Voltage Fluctuations	28

13.9	Appendix: Photographs of EUT	29

# General

## Certification of Accuracy of Test Data

The immunity tests which this report describes were conducted by an independent electromagnetic compatibility consultant, International Standards Laboratory in accordance with the EN55024:1998 / AS/NZS 4252.1:1994 which include EN61000-4 series regulations, Harmonic Current Emissions EN61000-3-2: 1995: A1:1998/A2:1998, and Voltage Fluctuations EN61000-3-3: 1995.

Equipment Tested:	Personal Computer
	Model/ Type/ Machine Type: VT3300
	Applied by Acer Inc.

Date of test:	2001/08/01

Test Engineer:	W.H. Chang

| Standard | Comment | Test Results |
| --- | --- | --- |
| EN61000-3-2: 1995 A1:1998/A2:1998 | Harmonic Current Emissions | Complies |
| EN61000-3-3: 1995 | Voltage Fluctuations | Complies |
| EN61000-4-2: 1995 | Electrostatic Discharge | Complies |
| EN61000-4-3: 1996 | Radio-Frequency, Electromagnetic Field | Complies |
| EN61000-4-4: 1995 | Electrical Fast Transient/Burst | Complies |
| EN61000-4-5: 1995 | Surge | Complies |
| EN61000-4-6: 1996 | Conductive Disturbance | Complies |
| EN61000-4-8: 1993 | Power Frequency Magnetic Field | Complies |
| EN61000-4-11: 1994 | Voltage Dips / Short Interruption and Voltage Variation | Complies |

Approve & Signature

---------------------------------------------

L. Y. Soong/Director

This test report accurately contains the test results of the above standards at the time of the test.

The results in this report apply only to the sample(s) tested.

This test report shall not be reproduced except in full, without the written approval of International Standards Laboratory.

# Summary

### Applicant Information

Applicant:	Acer Inc.
	7 Hsin Ann Rd., Science-Based Industrial Park
	Hsinchu 30077
	Taiwan, R. O. C.

### Operation Environment

Power supply : 	230 Vac / 50 Hz

## Description of Equipment Under Test

### Description of Support Equipment

- Description:	Acer Digital Camera
Model:	DVC-VII
Serial Number:	N/A
Power Supply Type:	From PC USB Port
Power Cord:	N/A
FCC ID:	(Comply with FCC DOC)

- Description:	Microsoft Joy Stick
Model Number:	90873
Serial Number:	01151545
Power Supply Type:	N/A
Power Cord:	N/A
FCC ID:	C3KMGP1

- Description:	Andrea Headphone
Model Number:	NC-80
Serial Number:	N/A
Power Supply Type:	N/A
Power Cord:	N/A
FCC ID:	N/A

- Description:	Koka  Headphone
Model Number:	ST-8
Serial Number:	N/A
Power Supply Type:	N/A
Power Cord:	N/A
FCC ID:	N/A

- Description:	KOKA Microphone
Model Number:	DM-510
Serial Number:	N/A
Power Supply Type:	N/A
Power Cord:	N/A
FCC ID:	N/A

- Description:	KOKA Microphone
Model Number:	DM-510
Serial Number:	N/A
Power Supply Type:	N/A
Power Cord:	N/A
FCC ID:	N/A

- Description:	SONY radio cassette player
Model Number:	WM-FX50
Serial Number:	N/A
Power Supply Type:	N/A
Power Cord:	N/A

- Description:	HP Printer
	(for parallel interface port)
Model Number:	2225C
Serial Number:	N/A
Power Supply Type:	Switching
	(AC to AC Xfmr, Wall Mounted Type)
Power Cord:	Nonshielded, Detachable
	With Grounding Pin
FCC ID:	DSI6XU2225

- Description:	Logitech Mouse
Model Number:	M-SAH
Serial Number:	LZB81251703
Power Supply Type:	N/A
Power Cord:	N/A
FCC ID:	DZL211029

- Description:	Aceex Modem
	(for serial interface port 1)
Model Number:	DM1414
Serial Number:	960063772
Power Supply Type:	Linear, Power Adapter
	( AC to AC Xfmr, Wall Mounted Type )
Power Cord:	Nonshielded, Without Grounding Pin
FCC ID:	IFAXDM1414

- Description:	Aceex Modem
	(for serial interface port 2)
Model Number:	DM1414
Serial Number:	960063771
Power Supply Type:	Linear, Power Adapter
	( AC to AC Xfmr, Wall Mounted Type )
Power Cord:	Nonshielded, Without Grounding Pin
FCC ID:	IFAXDM1414

- Description:	Acer Monitor
Model:	7377xe
Serial Number:	999027100501700055P644E1 P	
Power Supply Type:	Switching
Power Cord:	Nonshielded, Detachable
FCC ID:	(Comply with FCC DOC)

- Description:	Acer Keyboard
Model Number:	6511-TW4C
Serial Number:	916600704C83D11076S00000
Power Supply Type:	N/A
Power Cord:	N/A
FCC ID:	JVPKBS-WIN

- Description:	Personal Computer
Model:	IBM 2170
Serial No.:	N/A	
Power Supply Type :	Switching
	Delta  (Model: DPS-145PB-80A)
Hard Disk Drive:	Maxtor (Model: 91303D6) 13.3GB
Floppy Driver:	Panasonic (Model: JU256A276P ) 
CD-ROM Drive:	AOpen (Model: CD-940E/TKU PRO) 
ZIP Driver:	Iomega (Model:Z100ATAPI)
LAN Card	Accton (Model: EN1207D-TX1)
FDD/HDD Controller and	
VGA port/ Parallel/
Serial port:	Built on Motherboard
VGA port:	one 15-pin
Parallel Port:  	one 25-pin
Serial Port:  	one 9-pin
Keyboard Connector:  	6-pin
Mouse Connector: 	6-pin
USB Connector:  	two 4-pin
Game Port: 	one 15-pin
Speaker Port: 	one 
Microphone Port: 	one
Line In Port: 	one 
Power Cord:	Nonshielded, Detachable
FCC ID:	N/A (comply witch FCC DOC)

- Description:	Acer USB Keyboard
Model Number:	6511-UV
Serial Number:	N/A 
Power Supply Type:	N/A
Power Cord:	N/A
FCC ID:	N/A(comply with FCC DOC)

- Description:	Acer USB Mouse
Model:	M-U48a
Serial Number:	LZE01267035
Power Supply Type:	N/A
Power Cord:	N/A
FCC ID:	JNZ211360

- Description:	Acer USB Speaker
Model Number:	V-1
Serial Number:	N/A
Power Supply Type:	N/A
Power Cord:	N/A
FCC ID:	(Comply with FCC DOC)

### Software for Controlling Support Unit

A test program which generates a complete line of continuously repeating "H" pattern is used as the software test program.  The program was executed as follows:

- Read and write to the disk drives.

- Capture Video image from digital camera than playback to display.

- Send audio signal to the headphone.

- Send audio signal to the headphone.

- Receive audio signal from the microphone.

- Receive audio signal from the microphone.

- Receive audio signal from walkman.

- Send H pattern to the parallel port device (Printer).

- Send H pattern to the serial port device (Modem 1).

- Send H pattern to the serial port device (Modem 2).

- Send H pattern to the video port device (Monitor).

- Send signal form EUT to server through LAN port.

- Send signal form EUT to server through Telephone port.

- Repeat the above steps.

|  | Filename |  |  | Issued Date |
| --- | --- | --- | --- | --- |
| LAN | EMC.exe |  |  | 11/22/1996 |
| Monitor | HH.bat |  |  | 8/20/1991 |
| Modem 1 | Hm.bat |  |  | 8/20/1991 |
| Modem 2 | Hm.bat |  |  | 8/20/1991 |
| Printer1 | Wordpad.exe |  |  | 11/11/1999 |
| Digital Camera | Acer Cap.exe |  |  | 8/10/1998 |
| Telephone |  | Hypertrm.exe | 06/08/200 |  |

### I/O Cable Condition of EUT and Support Units

| Description | Path | Cable Length | Cable Type | Connector Type |
| --- | --- | --- | --- | --- |
| AC Power Cord | 110V (~240V) to AC Power Cord Inlet (3-pin) | 1.8M | Nonshielded, Detachable | Plastic Head Plastic Hood |
| Telephone Line | Server to EUT RJ 11 Connector | 1.2M | Nonshielded, Detachable to PC | Plastic Head Plastic Hood |
| Telephone Line | with 600 Ωdummy load | 1.2M | Nonshielded, Detachable to PC | Plastic Head Plastic Hood |
| Server Data Cable | Server to EUT LAN port | 33 feet | Nonshielded, Detachable | RJ-45, with Metal Head, Metal Hood |
| Keyboard Data Cable | Keyboard to PC Keyboard port | 1.8M | Shielded, Undetachable | Metal Head Plastic Hood |
| Monitor Data Cable | Monitor to PC VGA port | 1.6M | Shielded, Detachable | Metal Head Plastic Hood |
| Modem Data Cable | Modem to PC COM 1 port | 1.5M | Shielded, Detachable | Metal Head Metal Hood |
| Modem Data Cable | Modem to PC COM 1 port | 1.5M | Shielded, Detachable | Metal Head Metal Hood |
| Mouse Data Cable | Mouse to PC Mouse port | 1.8M | Shielded, Undetachable | Metal Head without Hood |
| Printer Data Cable | Printer to PC Parallel port | 1.5M | Shielded, Detachable | Metal Head Plastic Hood |
| Audio-in Data Cable | Walkman to PC Audio-In Port | 1.5M | Nonshielded, Detachable to PC | Metal Head Plastic Hood |
| Microphone Data Cable | Microphone to Mic Jack of PC | 1.5M | Nonshielded, Undetachable | Metal Head Plastic Hood |
| Microphone Data Cable | Microphone to Mic Jack of PC | 1.5M | Nonshielded, Undetachable | Metal Head Plastic Hood |
| Headphone Data Cable | Headphone to Line-out jack of PC | 1.5M | Nonshielded, Undetachable | Metal Head without Hood |
| Headphone Data Cable | Headphone to Line-out jack of PC | 1.5M | Nonshielded, Undetachable | Metal Head without Hood |
| Joy Stick Data Cable | Joystick to PC Joystick port | 1.2M | Shielded, Undetachable | Metal Head without Hood |
| USB CCD Data Cable | Digital camera to PC USB port | 1.6M | Shielded, Detachable | Metal Head Plastic Hood |

## Description of Equipment Under Test

EUT

Description:	Personal Computer
Condition:	Pre-Production
Model:	VT3300
Serial Number:	N/A
Power Supply Type:	Switching
	FSP(Model:FSP160-60SAV(PF)) 160W

Hard Disk Driver:	Seagate (Model: ST340810A) 40GB

Floppy Driver:	Panasomic(Model:JU-256A046P)

DVD-ROM Driver:	Pioneer (Model: DVD-116)

VGA Card:	LEADTEK(Model: Winfast Geforce 2 MX)

Modem Card:	ASKEY (Model: 1456VQH20E)

FDD/HDD Controller and

Parallel/Serial ports:	Built on Motherboard
Parallel Port:  	one 25-pin
Serial Port:  	two 9-pin

LAN Port: 	one 8-pin
Keyboard Connector:  	one 6-pin
Mouse Connector: 	one 6-pin
USB Connector:  	four 4-pin

Game Port:	one 15-pin

Speaker Port: 	two

Microphone Port: 	two

Line In Port: 	one

Power Cord:	Nonshielded, Detachable

Display:	CRT

Maximum Resolution: 	1280X1024

Speed and CPU

100MHz	Pentium 4-1.4,1.5,1.7,1.8,1.9,2.0GHz

All types of CPU, switching power supplies, hard disk drivers, cd-rom drivers, modem cards, and video modes listed above have been tested, we only present the worst case test data in the report. The test configuration is listed below:

CPU:	Pentium 4-2.0 GHz
Power supply:	FSP(Model:FSP160-60SAV(PF)) 160W
Hard disk driver:	Seagate (Model: ST340810A) 40GB

VGA Card:	LEADTEK(Model: Winfast Geforce 2 MX)

EMI Noise Source:
Crystal: 25MHz (X1), 14.318MHz (X2),32.768KHz (X3),24.576MHz(X4)
Clock Generator:U8

EMI Solution:

- Five spring fingers were added between add-on card bracket and metal enclosure

# Electrostatic discharge (ESD) immunity

## Electrostatic discharge (ESD) immunity test

Port:	Enclosure

Basic Standard:	EN61000-4-2

Requirements:	Air	 +/- 8 kV

Contact	 +/- 4 kV

Criteria:	B

Temperature:	25 degree C

Humidity:	59%

Test Setup

EUT is 1m from the wall and other metallic structure.

10cm

0.5m x 0.5m metal plane

EUT

0.5mm insulation

1.6m x 0.8m metal plane

Non-Metallic /

Non-Conducted Table

470K		80 cm

470K

Metal Full Soldered Ground Plane

Test Result

Performance of EUT complies with the given specification.

# Radio-Frequency, Electromagnetic Field immunity

## Radio-Frequency, Electromagnetic Field immunity test

Port:	Enclosure

Basic Standard:	EN61000-4-3

Modulation	AM 1KHz  80%

Requirements:	3 V/m

Frequency range:	80 MHz~1 GHz

Step:	1% of last step frequency

Step time:	800 ms

Polarization:	Vertical and Horizontal

Criteria:	A

Temperature:	26 degree C

Humidity:	46%

Test Setup

The field sensor is placed at one calibration grid point to check the intensity of the established fields on both polarizations. EUT is adjust to have each side of EUT face coincident with the calibration plane.  A CCD camera is used to monitor the condition of EUT for the performance judgment.

1 m

Anechoic Chamber

EUT

Non-Conducted

Table

To Amplifier output

80 cm		1.55m

To Remote Controller

Metal Full Soldered Ground Plane

Amplifier

Signal Generator

To Antenna

Test Result

Performance of EUT complies with the given specification.

# Electrical Fast transients/burst immunity

## Electrical Fast transient/burst immunity test

Port:	AC mains

Basic Standard:	EN61000-4-4

Requirements:	1 KV

Criteria:	B

Rise Time:	5ns

Hold Time:	50ns

Repetition Frequency:	5KHz

Temperature:	25 degree C

Humidity:	59%

Test Procedure

The EUT was setup on a nonconductive table 0.8 m above a reference ground plane.

|  | Voltage | 1 KV |  |
| --- | --- | --- | --- |
| Test Points | Polarity | Result | Comment |
| Line | + | N | 60 sec |
|  | - | N | 60 sec |
| Neutral | + | N | 60 sec |
|  | - | N | 60 sec |
| Ground | + | N | 60 sec |
|  | - | N | 60 sec |
| Line to | + | N | 60 sec |
| Neutral | - | N | 60 sec |
| Line to | + | N | 60 sec |
| Ground | - | N | 60 sec |
| Neutral to | + | N | 60 sec |
| Ground | - | N | 60 sec |
| Line to Neutral | + | N | 60 sec |
| to Ground | - | N | 60 sec |

Note:  ’N’  means normal, the EUT function is correct during the test.

Test Setup

EUT is at least 50cm from the conductive structure .

EUT

AC main

10cm insulation support

Non-Metallic /

Capacitive coupling	Non-Conducted Table

test generator

80 cm	clamp

AE

10cm

Metal Full Soldered Ground Plane      insulation support

Test Result

Performance of EUT complies with the given specification.

## Electrical Fast transient/burst immunity test for I/O cable

Port:	phone, twisted pairs LAN port

Basic Standard:	EN61000-4-4

Requirements:	0.5 kV

Criteria:	B

Rise Time:	5ns

Hold Time:	50ns

Repetition Frequency:	5KHz

Temperature:	25 degree C

Humidity:	59%

Test Result

Performance of EUT complies with the given specification.

# Surge Immunity

## Surge immunity test

Port:	AC mains and Telephone Line

Basic Standard:	EN61000-4-5

Criteria:	B

Rise Time:	1.2us

Hold Time:	50us

Temperature:	25 degree C

Humidity:	59%

Test Level:	AC Power Port: 1 kV (line to line), 2kV (line to earth)

Telephone Line: 1kV (line to ground)

Repetition Rate:	 30 second

Angle:	 0

 90

 270

Polarity:	 positive	 negative

Test Setup

AC power supply and

Voltage Supply to EUT

EUT

To the Peripherals

To AC main Supply	Non-Metallic /

Non-Conducted Table

80 cm

Transient-1000

Metal Full Soldered Ground Plane

Test Result

Performance of EUT complies with the given specification.

# Immunity to Conductive Disturbance

## Immunity to Conductive Disturbance

Port:	AC mains & Telephone ports & LAN port

Basic Standard:	EN61000-4-6

Test Level:	3 V

Modulation	AM 1KHz  80%

Criteria:	A

Frequency range:	0.15 MHz - 80MHz

Step:	1% of last Frequency

Step time:	1000 mS

Temperature:	25 degree C

Humidity:	59%

Test Setup

30mm	50

Amplifier

EUT

with earth terminal

CDN

30~50mm

To AC main 	To AC main

CDN

AE

Supply 	Supply

Signal

Generator

0.1 m support

Metallic

Plane

Non-Metallic /	1m x 1m

Non-Conducted Table	metal

ground

80 cm	plane

This Metal Plane is 50cm

from the wall

Metal Full Soldered Ground Plane

Test Result

Performance of EUT complies with the given specification.

# Power Frequency Magnetic Field immunity

## Power Frequency Magnetic field immunity test

Port:	Enclosure

Basic Standard:	EN61000-4-8

Requirements:	1 A/m

Criteria:	A

Orientation:	X, Y, Z

Temperature:	25 degree C

Humidity:	59%

Test Setup

3m

Magnetic Field Antenna (1m x 1m)

To AC	To the

EUT

Power 	Peripheral	metal ground

To AC main 	Source	plane

Supply

Non-Metallic /

Non-Conducted Table

80 cm	 insulation support

Transient 1000

Ground

Wire

Metal Full Soldered Ground Plane

Test Result

Performance of EUT complies with the given specification.

# Voltage Dips, Short Interruption and Voltage Variation immunity

## Voltage Dips, Short Interruption and Voltage Variation immunity test

Port:	AC mains

Basic Standard:	EN61000-4-11

Phase:	0

Test intervals:	3 times with 10s each

Voltage Dips:	>95% in 0.5 period

Criteria:	B

Voltage Dips:	30% in 25 period

Criteria:	C

Voltage Interruption:	>95% in 250 period

Criteria:	C

Test Setup

AC power supply to EUT

Interruptions & Voltage

Dips Control

To AC main		To the Peripherals

EUT

Supply

Non-Metallic /

Non-Conducted Table

Transient-1000

80 cm

Metal Full Soldered Ground Plane

Test Result

Performance of EUT complies with the given specification.

# Harmonics

## Harmonics test

Port:	AC mains

Active Input Power:	>75W

Basic Standard:	EN61000-3-2

Test Duration:	3min.

Class:	D

Test Procedure

The EUT is supplied in series with shunts or current transformers from a source having the same nominal voltage and frequency as the rated supply voltage and frequency of the EUT.  The EUT is configured to its rated current with additional resistive load when the testing is performed.

Equipment having more than one rated voltage shall be tested at the rated voltage producing the highest harmonics as compared with the limits.

Result

Performance of EUT complies with the given specification.

Test Data

| Power (W) | Power Factor | Power Voltage | Power Current |
| --- | --- | --- | --- |
| 156.474 | 0.773 | 229.398 | 0.733 |

| Maximum permissible harmonic current (A) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Order | Data | Order | Data | Order | Data |
| 1 |  | 16 | 0.0003 | 31 | 0.0031 |
| 2 | 0.0019 | 17 | 0.0079 | 32 | 0.0003 |
| 3 | 0.4618 | 18 | 0.0003 | 33 | 0.0023 |
| 4 | 0.0008 | 19 | 0.0070 | 34 | 0.0005 |
| 5 | 0.1591 | 20 | 0.0003 | 35 | 0.0025 |
| 6 | 0.0004 | 21 | 0.0048 | 36 | 0.0005 |
| 7 | 0.0555 | 22 | 0.0003 | 37 | 0.0021 |
| 8 | 0.0003 | 23 | 0.0048 | 38 | 0.0002 |
| 9 | 0.0252 | 24 | 0.0003 | 39 | 0.0017 |
| 10 | 0.0004 | 25 | 0.0037 | 40 | 0.0002 |
| 11 | 0.0218 | 26 | 0.0003 | 41 |  |
| 12 | 0.0003 | 27 | 0.0036 | 42 |  |
| 13 | 0.0134 | 28 | 0.0003 | 43 |  |
| 14 | 0.0003 | 29 | 0.0030 | 44 |  |
| 15 | 0.0132 | 30 | 0.0003 | 45 |  |

Test Data

| Power (W) | Power Factor | Power Voltage | Power Current |
| --- | --- | --- | --- |
| 156.474 | 0.773 | 229.398 | 0.733 |

| Maximum permissible harmonic current per watt (mA/W) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Order | Data | Order | Data | Order | Data |
| 3 | 2.9595 | 17 | 0.0506 | 31 | 0.0197 |
| 5 | 1.0213 | 19 | 0.0448 | 33 | 0.0146 |
| 7 | 0.3556 | 21 | 0.0306 | 35 | 0.0158 |
| 9 | 0.1617 | 23 | 0.0306 | 37 | 0.0132 |
| 11 | 0.1399 | 25 | 0.0234 | 39 | 0.0108 |
| 13 | 0.0861 | 27 | 0.0232 |  |  |
| 15 | 0.0845 | 29 | 0.0195 |  |  |

# Voltage Fluctuations

## Voltage Fluctuations test

Port:	AC  mains

Basic Standard:	EN61000-3-3

Observation period:	For Pst 10min

For Plt 2 hours

Test Procedure

The EUT is supplied in series with reference impedance from a power source with the voltage and frequency as the nominal supply voltage and frequency of the EUT.

Result

Performance of EUT complies with the given specification.

# Test Equipment List

| Location Remark | Equipment | Brand | Model | Start Service Date | Last Cal. Date | Next Cal. Date |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2,4,5,8,11 | Test Generator | PRECISION | TRA1000-126 S/N: TRA1H01B | 6/30/1994 | 8/9/2000 |  | 8/9/2001 |
| 3,6 | Power meter | HP | 438A S/N: 3513U06187 | 10/23/1997 | 10/23/2000 |  | 10/23/2001 |
| 3,6 | Power Sensor | HP | 8482A S/N: 3318A29614 | 10/23/1997 | 1/03/2000 |  | 1/03/2001 |
| 3 | Signal Generator | HP | 8648B S/N: 3642U01040 | 6/30/1997 | 3/13/2001 |  | 3/13/2003 |
| 3 | BILOG Antenna | CHASE | CBL6112 S/N: 2077 | 10/20/1998 | 1/10/2001 |  | 1/09/2002 |
| 3 | Power Amplifier | Amplifier Research | 100W1000M1 S/N: | 3/31/1996 | N/A |  | N/A |
| 3 | Field Strength Sensor | Amplifier Research | FP2000 S/N: 15397 | 3/31/1996 | N/A |  | N/A |
| 3 | Field Strength Meter | Amplifier Research | FM2000 S/N: A285000011 | 3/31/1996 | N/A |  | N/A |
| 3 | Thermo-hygrometer | CRECER | S/N: ISL-C-003 | 11/26/1999 | 1/12/2001 |  | 1/11/2002 |
| 4 | Clamp | Precision | 1604242 S/N: CNEFT1000-103 | 6/30/1994 | N/A |  | N/A |
| 5,6 | CDN | FCC Inc. | FCC-801-T2 S/N:9720 | 6/30/1994 | 1/11/2001 |  | 1/10/2002 |
| 6 | CDN | FCC Inc. | FCC-801-T4 S/N: 9721 | 6/30/1994 | 1/11/2001 |  | 1/10/2002 |
| 6 | CDN | FCC Inc. | FCC-801-M3-25A S/N: 2032 | 6/30/1994 | 1/11/2001 |  | 1/10/2002 |
| 6 | Signal Generator | HP | 8656B S/N: 2635A04675 | 6/30/1992 | 8/17/2000 |  | 8/17/2001 |
| 6 | Power Amplifier | Amplifier Research | 150A100 S/N: 1-1-R-02157 | 3/1/1996 | N/A |  | N/A |
| 6 | Passive Impedance Adapter | FCC Inc. | FCC-801-150-50-CDN S/N: 9758 & 9759 | 3/1/1996 | N/A |  | N/A |
| 6 | 50 ohms load | Weinschel Corp | 1429-4 S/N: DB3318 | 3/1/1996 | N/A |  | N/A |
| 6 | 6dB Attenuator | Weinschel Corp | 33-6-34 S/N: BC5975 | 3/1/1996 | N/A |  | N/A |
| 8 | Magnetic Field Antenna | Precision | TRAIZ44B S/N: MF1000-23 | 8/30/1997 | N/A |  | N/A |
| 8 | Clamp Meter | TES | 3090 S/N: 990900322 | 5/09/2001 | 5/09/2001 |  | 5/09/2002 |
| *2, *3 | 3-Channel Power Analysis System | Xitron Technologies | 2503AH S/N: 25015940001 | 6/30/1994 | 2/09/2001 |  | 2/09/2002 |
| *2, *3 | Frequency Converter | Extech Electronics | CFC-110 | 6/30/1994 | N/A |  | N/A |
| 2,4,5,6,8,11 | Thermo-hygrometer | MICROLIFE | S/N: ISL-C-004 | 11/26/1999 | 1/12/2001 |  | 1/12/2002 |

Note: ***: The equipment is sent for calibration.

Location Remark List:

2: EN61000-4-2	3: EN61000-4-3	4: EN61000-4-4	5: EN61000-4-5

6: EN61000-4-6	8: EN61000-4-8	11: EN61000-4-11

*2: EN61000-3-2	*3:EN61000-3-3

## Software for Controlling Spectrum/Receiver and Calculating Test Data

| Radiation/Conduction |  | Filename | Version |
| --- | --- | --- | --- |
| EN61000-3-2 | IEC1000.EXE |  | 1.0F |
| EN61000-3-3 | IEC1000.EXE |  | 1.0F |
| EN61000-4-3 | Tile.Exe |  | 1.12.E |
| EN61000-4-6 | Tile.Exe |  | 1.11.J |
| EN61000-4-2 | Tracs.Exe |  | 2.0 |
| EN61000-4-4 | Tracs.Exe |  | 2.0 |
| EN61000-4-5 | Tracs.Exe |  | 2.0 |
| EN61000-4-8 | Tracs.Exe |  | 2.0 |
| EN61000-4-11 | Tracs.Exe |  | 2.0 |

# Photographs

## Photos of ESD measurement

## Photos of RF Field Strength Susceptibility Measurement

.

## Photos of Electrical Fast Transient/Burst measurement

## Photos of Surge measurement

## Photo of Conductive Measurement

## Photo of Magnetic field measurement

## Photos of Voltage Dips measurement

## Photos of Harmonics and Voltage Fluctuations

## Appendix: Photographs of EUT

Please find this appendix in the File of ISL-01A063P