EN55022 / CISPR 22 / AS/NZS 3548

Class B

EMI TEST  REPORT

of

Personal Computer

Model/ Type/ Machine Type

VT7500; VT7500G

Applied by:

Acer Inc.

7F, No. 137, Sec. 2, Chien-Kuo N. Rd.,

Taipei 104,

Taiwan, R. O. C.

Test Performed by:

International Standards Laboratory

No. 120, Lane 180, San Ho Tsuen, Hsin Ho Rd.
Lung-Tan Hsiang, Tao Yuan County 325
Taiwan, R.O.C.
Tel:(03)407-1718  Fax:(03)407-1738

Contents of Report

1.	General	1

1.1	Certification of Accuracy of Test Data	1

1.2	Summary	2

1.2.1	Description of Equipment Under Test (EUT)	2

1.2.2	Description of EUT and Support Equipment Included in Tests	2

1.2.3	Test Procedure and Specification	3

1.2.4	Tests Performed	3

2.	Powerline Conducted Emissions	4

2.1	Configuration and Procedure	4

2.1.1	EUT Configuration	4

2.1.2	Test Procedure	4

2.1.3	EMI Receiver/Spectrum Analyzer Configuration (for the frequencies tested)	4

2.2	Test Data: CPU: Pentium 4- 2.4 GHz, Speed: 133MHz, SPS: Delta (Model:DPS-200PB- 112A), Modem Card: Ambit V92 56K HSFI(Model: T62M206.00), Maximum Resolution: 1280X1024.	5

3.	Open Field Radiated Emissions	7

3.1	Configuration and Procedure	7

3.1.1	EUT Configuration	7

3.1.2	Test Procedure	7

3.1.3	EMI Receiver/Spectrum Analyzer Configuration (for the frequencies tested)	7

3.2	Test Data:.CPU: Pentium 4- 2.4 GHz, Speed: 133MHz, SPS: Delta (Model:DPS-200PB- 112A), Modem Card: Ambit V92 56K HSFI(Model: T62M206.00), Maximum Resolution:  1280X1024.	8

4.	Appendix	10

4.1	Appendix A: Measurement Procedure for Powerline Conducted Emissions	10

4.2	Appendix B: Test Procedure for Radiated Emissions	11

4.3	Appendix C: Test Equipment	12

4.3.1	Test Equipment List	12

4.3.2	Software for Controlling Spectrum/Receiver and Calculating Test Data	13

4.4	Appendix D: Layout of EUT and Support Equipment	14

4.4.1	General Power Main Port Conducted Test Configuration	14

4.4.2	General Radiation Test Configuration	15

4.5	Appendix E: Description of Support Equipment	16

4.5.1	Description of Support Equipment	16

4.5.2	Software for Controlling Support Unit	20

4.5.3	I/O Cable Condition of EUT and Support Units	21

4.6	Appendix F: Description of Equipment Under Test	23

4.7	Appendix G: Uncertainty of Measurement	25

4.8	Appendix H: Photographs of EUT Configuration Test Set Up	27

# General

## Certification of Accuracy of Test Data

The electromagnetic interference tests which this report describes were conducted by an independent electromagnetic compatibility consultant, International Standards Laboratory in accordance with the EN55022:1994 /A1:1995 /A2: 1997; CISPR 22:1993/A1:1995/A2:1996; AS/NZS 3548:1995 /A1:1997 /A2:1997 test Procedure.  There is no deviation on test method of EN55022:1994 /A1:1995 /A2: 1997; CISPR 22:1993/A1:1995/A2:1996; AS/NZS 3548:1995 /A1:1997 /A2:1997 test Procedure.

This test report accurately represents the test results generated by sample equipment under test at the time of the test.

Equipment Tested:		Personal Computer

Model/ Type/ Machine Type: VT7500; VT7500G

Applied by Acer Inc.

Sample received Date:	2002/04/11

Final test Date :		2002/04/26

Temperature	24ºC(Conduction Test);	21ºC (Radiation Test)

Humidity:	56% (Conduction Test); 	61% (Radiation Test)

Test Engineer:	Chance Chan

Test Site: 		Chamber 02; Conduction 02

The results show that the sample equipment tested as described in this report is in compliance with the Class B conducted and radiated emission limits of EN55022:1994 /A1:1995 /A2: 1997; CISPR 22:1993/A1:1995/A2:1996; AS/NZS 3548:1995 /A1:1997 /A2:1997 regulations.

Approve & Signature

---------------------------------------

L. Y. Soong/Director

Test results given in this report  apply only to the specific sample(s) tested under stated test conditions.

This report shall  not be reproduced other than in full without the explicit  written consent of ISL. This report totally contains 30 pages, including 1 cover page , 1 contents page, and 28 pages for the test description.

Note: This test report shall not be reproduced except in full, without the written approval of International Standards Laboratory.

## Summary

### Description of Equipment Under Test (EUT)

Description:		Personal Computer

Model/ Type/ Machine Type:	VT7500; VT7500G

Applicant:		Acer Inc.
		7F, No. 137, Sec. 2, Chien-Kuo N. Rd.,
		Taipei 104,
		Taiwan, R. O. C.

A more detailed, technical description of the EUT is contained in Appendix F

### Description of EUT and Support Equipment Included in Tests

The EUT is a Personal Computer (Model/ Type/ Machine Type: VT7500; VT7500G), which was tested with the following support units:

- Logitech USB Mouse	Model: M-u48a

- Logitech USB Mouse	Model: M-u48a

- Logitech USB Mouse	Model: M-u48a

- KOKA Microphone	Model: DM-510

- Firstline Headphone	Model: H1160.0

- Firstline Headphone	Model: H1160.0

- Broadband Router	Model: Router 904

- Acer Digital Camera	Model: DVC-VII

- Logitech USB Mouse	Model: M-u48a

- Firstline Headphone	Model: H1160.0

- KOKA Microphone	Model: DM-510

- Coson radio cassette player	Model: C-2087

- HP Printer	Model: C2642A

- Acer Mouse	Model: M-S42

- Acer Monitor	Model: G781

- Acer Keyboard	Model: 6511-TW4C

- Acer USB Keyboard	Model: 6511-UV

- Acer Personal Computer	Model: VT7200

- Acer Speaker	Model: A201S

A more detailed technical description of the support equipment is contained in Appendix E.

### Test Procedure and Specification

The tests were performed in accordance with EN55022:1994 /A1:1995 /A2: 1997; CISPR 22:1993/A1:1995/A2:1996; AS/NZS 3548:1995 /A1:1997 /A2:1997 regulations as detailed in the Appendix A & B and the individual test sections. The test instrument used is detailed in Appendix A. The specification used was the Class B limits of EN55022:1994 /A1:1995 /A2: 1997; CISPR 22:1993/A1:1995/A2:1996; AS/NZS 3548:1995 /A1:1997 /A2:1997 regulations.

### Tests Performed

- Power main port conducted emissions in shielded room.
See Part 2 of this report for details.

- Telecommunication port conducted emissions in shielded room
See Part 3 of this report for details.

- Radiated emissions in 10-meter EMC chamber.
See Part 4 of this report for details.

# Powerline Conducted Emissions

## Configuration and Procedure

### EUT Configuration

The conducted emission test setups are in accordance with Figs 9, 10(a) and 10(b) of ANSI C63.4-1992, CFR 47 Part 15 Subpart B; or EN55022:1994 /A1:1995 /A2: 1997; CISPR 22:1993/A1:1995/A2:1996.

The EUT was set up on the non-conductive table which is 1.0 by 1.5 meter, 80cm above ground.  The wall of the shielded room was located 40cm to the rear of the EUT.

Power to the EUT was provided through the LISN. The impedance vs frequency characteristic of the LISN is complied with the limit shown on the figure 1 of ANSI C63.4-1992, CFR 47 Part 15 Subpart B; or EN55022:1998; CISPR 22:1997

Both lines (neutral and hot) were connected to the LISN in series at testing. A coaxial-type connector which provides a 50 OHM terminating impedance was provided for connecting the test instrument. The excess length of the power cord was folded back and forth at the center of the lead so as to form a bundle not exceeding 40cm in length.

Any changes made to the configuration, or modifications made to the EUT, during testing are noted in the following test record.

If the EUT is a Personal Computer or a peripheral of personal computer, and the personal computer has an auxiliary AC outlet which can be used for providing power to an external monitor, then all measurements will be made with the monitor power from first the computer-mounted AC outlet and then a floor-mounted AC outlet.

### Test Procedure

The system was set up as described above, with the EMI diagnostic software running. The powerline conducted EMI tests were run on the hot and neutral conductors of the power cord and the results were recorded.

The highest emissions were analyzed in details by operating the spectrum analyzer in fixed tuned mode to determine the nature of the emissions and to provide information which could be useful in reducing their amplitude.

### EMI Receiver/Spectrum Analyzer Configuration (for the frequencies tested)

Frequency Range:	150KHz--30MHz
Detector Function:	Quasi-Peak / Average Mode

Bandwidth (RBW):	9KHz

## Test Data: CPU: Pentium 4- 2.4 GHz, Speed: 133MHz, SPS: Delta (Model:DPS-200PB- 112A), Modem Card: Ambit V92 56K HSFI(Model: T62M206.00), Maximum Resolution: 1280X1024.

Table 2.2.1 Power Line Conducted Emissions (Hot)

|  | Corrective Factor |  |  |  | Quasi-Peak |  |  |  |  | Average |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency (MHz) | LISN Loss (dB) |  | Cable Loss (dB) |  | Corrected Amplitude (dBuV) | Limit (dBuV) | Margin (dB) |  |  | Corrected Amplitude (dBuV) |  | Limit (dBuV) |  | Margin (dB) |  |
| 0.21016 |  | 0.10 | 0.14 | 47.31 |  | 64.28 |  | -16.97 | 46.82 |  | 54.28 |  | -7.46 |  |  |
| 0.31626 |  | 0.10 | 0.16 | 38.96 |  | 61.25 |  | -22.29 | 37.69 |  | 51.25 |  | -13.56 |  |  |
| 0.42161 |  | 0.10 | 0.17 | 27.88 |  | 58.24 |  | -30.36 | 25.24 |  | 48.24 |  | -23.00 |  |  |
| 0.63373 |  | 0.14 | 0.16 | 36.94 |  | 56.00 |  | -19.06 | 34.40 |  | 46.00 |  | -11.60 |  |  |
| 0.74008 |  | 0.16 | 0.16 | 28.28 |  | 56.00 |  | -27.72 | 26.60 |  | 46.00 |  | -19.40 |  |  |
| 6.44817 |  | 0.34 | 0.35 | 36.81 |  | 60.00 |  | -23.19 | 23.20 |  | 50.00 |  | -26.80 |  |  |
| 6.56766 |  | 0.34 | 0.35 | 37.33 |  | 60.00 |  | -22.67 | 22.86 |  | 50.00 |  | -27.14 |  |  |
| 6.76322 |  | 0.35 | 0.36 | 38.92 |  | 60.00 |  | -21.08 | 23.53 |  | 50.00 |  | -26.47 |  |  |
| 7.10847 |  | 0.35 | 0.38 | 40.05 |  | 60.00 |  | -19.95 | 24.30 |  | 50.00 |  | -25.70 |  |  |
| 7.5831 |  | 0.36 | 0.41 | 35.27 |  | 60.00 |  | -24.73 | 21.54 |  | 50.00 |  | -28.46 |  |  |

* NOTE:	Margin = Corrected Amplitude - Limit

Corrected Amplitude = Receiver Reading + LISN Loss + Cable Loss

A margin of -8dB means that the emission is 8dB below the limit

Tested by:

_________________________

Chance Chan

Table 2.2.2 Power Line Conducted Emissions (Neutral)

|  | Corrective Factor |  |  |  | Quasi-Peak |  |  |  |  | Average |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency (MHz) | LISN Loss (dB) |  | Cable Loss (dB) |  | Corrected Amplitude (dBuV) | Limit (dBuV) |  | Margin (dB) |  | Corrected Amplitude (dBuV) | Limit (dBuV) |  | Margin (dB) |
| 0.21181 |  | 0.11 | 0.14 | 42.03 |  | 64.23 | -22.21 |  | 41.63 |  | 54.23 | -12.61 |  |
| 0.31768 |  | 0.16 | 0.16 | 31.91 |  | 61.21 | -29.30 |  | 30.05 |  | 51.21 | -21.16 |  |
| 0.42588 |  | 0.20 | 0.17 | 23.78 |  | 58.12 | -34.34 |  | 18.93 |  | 48.12 | -29.18 |  |
| 0.63673 |  | 0.20 | 0.16 | 38.61 |  | 56.00 | -17.39 |  | 35.82 |  | 46.00 | -10.18 |  |
| 0.74308 |  | 0.20 | 0.16 | 28.05 |  | 56.00 | -27.95 |  | 25.71 |  | 46.00 | -20.29 |  |
| 6.45214 |  | 0.30 | 0.35 | 36.91 |  | 60.00 | -23.09 |  | 23.07 |  | 50.00 | -26.93 |  |
| 6.56083 |  | 0.30 | 0.35 | 37.57 |  | 60.00 | -22.43 |  | 22.90 |  | 50.00 | -27.10 |  |
| 6.75115 |  | 0.30 | 0.36 | 39.42 |  | 60.00 | -20.58 |  | 23.59 |  | 50.00 | -26.41 |  |
| 7.10585 |  | 0.30 | 0.38 | 39.52 |  | 60.00 | -20.48 |  | 25.64 |  | 50.00 | -24.36 |  |
| 7.61176 |  | 0.30 | 0.41 | 36.48 |  | 60.00 | -23.52 |  | 28.09 |  | 50.00 | -21.91 |  |

* NOTE:	Margin = Corrected Amplitude - Limit

Corrected Amplitude = Receiver Reading + LISN Loss + Cable Loss

A margin of -8dB means that the emission is 8dB below the limit

Tested by:

_________________________

Chance Chan

# Open Field Radiated Emissions

## Configuration and Procedure

### EUT Configuration

The radiated emissions test setups are in accordance with Figs 10(c) and 10(d) of ANSI C63.4-1992, CFR 47 Part 15 Subpart B; or EN55022:1994 /A1:1995 /A2: 1997; CISPR 22:1993/A1:1995/A2:1996.

The equipment under test was set up on the non-conductive table 80cm above ground of 10 meter open test site or 10 meter EMC chamber, same as conducted Excess data cable was folded back and forth to form a 30cm by 40cm bundle.

Any changes made to the configuration, or modifications made to the EUT, during testing are noted in the following test record.

If the EUT is a Personal Computer or a peripheral of personal computer, and the personal computer has an auxiliary AC outlet which can be used for providing power to an external monitor, then all measurements will be made with the monitor power from first the computer-mounted AC outlet and then a floor-mounted AC outlet.

### Test Procedure

The system was set up as described above, with the EMI diagnostic software running. We found the maximum readings by varying the height of antenna and then rotating the turntable. Both polarization of antenna, horizontal and vertical, are measured.

The highest emissions between 30 MHz to 1000 MHz were also analyzed in details by operating the spectrum analyzer and/or EMI receiver in quasi-peak mode to determine the precise amplitude of the emissions. While doing so, the interconnecting cables and major parts of the system were moved around, the antenna height was varied between one and four meters, its polarization was  varied between vertical and horizontal, and the turntable was slowly rotated, to maximize the emission.  The highest emissions of frequency bigger than 1000 MHz was analyzed in peak mode and/or average mode to determine the precise amplitude of the emission.

### EMI Receiver/Spectrum Analyzer Configuration (for the frequencies tested)

Frequency Range A:	30MHz--1000MHz
	Detector Function:	Quasi-Peak Mode

Bandwidth (RBW):	120KHz

Frequency Range B:	1000MHz--18000MHz
	Detector Function:	Peak Mode/Average mode

Bandwidth (RBW):	1 MHz

## Test Data:.CPU: Pentium 4- 2.4 GHz, Speed: 133MHz, SPS: Delta (Model:DPS-200PB- 112A), Modem Card: Ambit V92 56K HSFI(Model: T62M206.00), Maximum Resolution: 
1280X1024.

Table 3.2.1 Open Field Radiated Emissions (Horizontal)

| Meter Reading |  |  | Correction Factor |  |  |  |  |  |  | Corrected Emissions |  |  |  |  | Antenna |  | Turntable |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Freq. (MHz) | Ampl. (dBuV) |  | Ant. (dB/m) |  | Cable (dB) |  | Pre-Ampl. (dB) |  |  | Ampl. (dBuV/m) | Limit (dBuV/m) |  |  | Margin* (dB) | Height (cm) |  | Position (°) |  |
| 149.31 |  | 13.79 |  | 9.76 |  | 1.39 |  | 0.00 | 24.94 |  |  | 30.00 | -5.06 |  |  | 399.00 |  | 21.00 |
| 178.41 |  | 15.96 |  | 9.03 |  | 1.52 |  | 0.00 | 26.52 |  |  | 30.00 | -3.48 |  |  | 400.00 |  | 0.00 |
| 224.97 |  | 16.15 |  | 8.80 |  | 1.66 |  | 0.00 | 26.60 |  |  | 30.00 | -3.40 |  |  | 349.00 |  | 201.00 |
| 294.81 |  | 18.03 |  | 13.55 |  | 1.90 |  | 0.00 | 33.48 |  |  | 37.00 | -3.52 |  |  | 300.00 |  | 142.00 |
| 399.57 |  | 15.16 |  | 16.28 |  | 2.17 |  | 0.00 | 33.61 |  |  | 37.00 | -3.39 |  |  | 299.00 |  | 241.00 |
| 450.01 |  | 12.65 |  | 16.50 |  | 2.36 |  | 0.00 | 31.51 |  |  | 37.00 | -5.49 |  |  | 200.00 |  | 101.00 |
| 549.92 |  | 9.59 |  | 18.70 |  | 2.84 |  | 0.00 | 31.13 |  |  | 37.00 | -5.87 |  |  | 149.00 |  | 241.00 |
| 699.3 |  | 9.95 |  | 19.30 |  | 3.18 |  | 0.00 | 32.44 |  |  | 37.00 | -4.56 |  |  | 149.00 |  | 181.00 |
| 749.74 |  | 6.87 |  | 20.49 |  | 3.66 |  | 0.00 | 31.03 |  |  | 37.00 | -5.97 |  |  | 149.00 |  | 181.00 |
| 837.04 |  | 8.42 |  | 20.84 |  | 3.42 |  | 0.00 | 32.69 |  |  | 37.00 | -4.31 |  |  | 100.00 |  | 181.00 |

* NOTE:	Margin = Corrected Amplitude – Limit

Corrected Amplitude = Radiated Amplitude + Antenna Correction Factor + Cable Loss – Pre-Amplifier Gain

A margin of -8dB means that the emission is 8dB below the limit

BILOG Antenna	Distance:  10 meter, 	Frequency: under 1000MHz

Horn Antenna	Distance:  3 meter, 	Frequency: 1000MHz—18GHz

Tested by:

_________________________

Chance Chan

Table 3.2.2 Open Field Radiated Emissions (Vertical)

| Meter Reading |  |  | Correction Factor |  |  |  |  |  | Corrected Emissions |  |  |  |  |  | Antenna | Turntable |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Freq. (MHz) | Ampl. (dBuV) |  | Ant. (dB/m) |  | Cable (dB) |  | Pre-Ampl. (dB) |  | Ampl. (dBuV/m) | Limit (dBuV/m) |  | Margin* (dB) |  | Height (cm) |  | Position (°) |
| 114.39 |  | 12.64 |  | 11.84 |  | 1.23 |  | 0.00 | 25.72 |  | 30.00 |  | -4.28 |  | 100.00 | 101.00 |
| 125.06 |  | 12.04 |  | 11.70 |  | 1.30 |  | 0.00 | 25.04 |  | 30.00 |  | -4.96 |  | 149.00 | 241.00 |
| 165.8 |  | 14.01 |  | 9.16 |  | 1.48 |  | 0.00 | 24.64 |  | 30.00 |  | -5.36 |  | 100.00 | 282.00 |
| 170.65 |  | 15.27 |  | 9.19 |  | 1.51 |  | 0.00 | 25.97 |  | 30.00 |  | -4.03 |  | 100.00 | 282.00 |
| 249.22 |  | 17.85 |  | 11.59 |  | 1.76 |  | 0.00 | 31.21 |  | 37.00 |  | -5.79 |  | 100.00 | 321.00 |
| 296.75 |  | 15.69 |  | 13.57 |  | 1.90 |  | 0.00 | 31.16 |  | 37.00 |  | -5.84 |  | 100.00 | 161.00 |
| 499.48 |  | 11.23 |  | 17.69 |  | 2.69 |  | 0.00 | 31.61 |  | 37.00 |  | -5.39 |  | 99.00 | 61.00 |
| 699.3 |  | 9.37 |  | 19.30 |  | 3.18 |  | 0.00 | 31.85 |  | 37.00 |  | -5.15 |  | 200.00 | 261.00 |
| 782.72 |  | 7.65 |  | 20.43 |  | 3.30 |  | 0.00 | 31.38 |  | 37.00 |  | -5.62 |  | 149.00 | 81.00 |
| 837.04 |  | 7.89 |  | 20.84 |  | 3.42 |  | 0.00 | 32.15 |  | 37.00 |  | -4.85 |  | 198.00 | 161.00 |
| 900.09 |  | 8.69 |  | 20.90 |  | 3.34 |  | 0.00 | 32.93 |  | 37.00 |  | -4.07 |  | 198.00 | 0.00 |

* NOTE:	Margin = Corrected Amplitude – Limit

Corrected Amplitude = Radiated Amplitude + Antenna Correction Factor + Cable Loss - Pre-Amplifier Gain

A margin of -8dB means that the emission is 8dB below the limit

BILOG Antenna	Distance:  10 meter, 	Frequency: under 1000MHz

Horn Antenna	Distance:  3 meter, 	Frequency: 1000MHz—18GHz

Tested by:

_________________________

Chance Chan

# Appendix

## Appendix A: Measurement Procedure for Powerline Conducted Emissions

The EUT is set up in accordance with the suggested configuration given in ANSI C63.4-1992, CFR 47 Part 15 Subpart B; or EN55022:1998; CISPR 22:1997.  The measurements are performed in a 3 x 3 x 2.3 (m) Conduction test site. The EUT was placed on a non-conduction 1.0 x 1.5 (m) table, which is 0.8 meters above an earth-grounded.

Power to the EUT was provided through the LISN which has the Impedance vs. Frequency Characteristic in accordance with the Figure 1 of the ANSI C63.4-1992, CFR 47 Part 15 Subpart B; or EN55022:1994 /A1:1995 /A2: 1997; CISPR 22:1993/A1:1995/A2:1996. Power to the LISNs were filtered to eliminate ambient signal interference and these filters were bonded to the ground plane. Peripheral equipment required to provide a functional system (support equipment) for EUT testing was powered from the second LISN through a ganged, metal power outlet box which is bonded to the ground plane at the LISN.

If the EUT is supplied with a flexible power cord, the power cord length in excess of the distance separating the EUT from the LISN shall be folded back and forth at the center of the lead so as to form a bundle not exceeding 40cm in length.  If the EUT is provided with a permanently coiled power cord, bundling of the cord is not required. If the EUT is supplied without a power cord, the EUT shall be connected to the LISN by a power cord of the type specified by the manufacturer which shall not be longer than 1 meter. The excess power cord shall be bundled as described above. If a non-flexible power cord is provided with the EUT, it shall be cut to the length necessary to attach the EUT to the LISN and shall not be bundled.

The interconnecting cables were arranged and moved to get the maximum according to ANSI C63.4-1992, CFR 47 Part 15 Subpart B; or EN55022:1998; CISPR 22:1997.Both the line of power cord, hot and neutral, were measured.

The highest emissions were analyzed in details by operating the spectrum analyzer in fixed tuned mode to determine the nature of the emissions and to provide information which could be useful in reducing their amplitude.

## Appendix B: Test Procedure for Radiated Emissions

Preliminary Measurements in the Anechoic Chamber

The radiated emissions are initially measured in the anechoic chamber at a measurement distance of 3 meters. Desktop EUTs are placed on a wooden stand 0.8 meter in height. The measurement antenna is 3 meters from the EUT.  The test setup in anechoic chamber is the same as open site.   The turntable rotated 360°C.  The antenna height is varied from 1-2.5m.   The primary objective of the radiated measurements in the anechoic chamber is to identify the frequency spectrum in the absence of the electromagnetic environment existing on the open test site. The frequencies can then be preselected on the open test site to obtain the corresponding amplitude.  The initial scan is made with the spectrum analyzer in automatic sweep mode. The spectrum peaks are then measured manually to determine the exact frequencies.

Measurements on the Open Site or 10m EMC Chamber

The radiated emissions test will then be repeated on the open site or 10m EMC chamber to measure the amplitudes accurately and without the multiple reflections existing in the shielded room. The EUT and support equipment are set up on the turntable of one of the 3 or 10 meter open field sites. Desktop EUTs are set up on a wooden stand 0.8 meter above the ground.

For the initial measurements, the receiving antenna is varied from 1-4 meter height and is changed in the vertical plane from vertical to horizontal polarization at each frequency. Both reading are recorded with the quasi-peak detector and with the 120 kHz bandwidth. For frequency between 30 MHz and 1000MHz, the reading are recorded with peak detector and/or average detector and with the 1 MHz bandwidth.

At the highest amplitudes observed, the EUT is rotated in the horizontal plane while changing the antenna polarization in the vertical plane to maximize the reading. The interconnecting cables were arranged and moved to get the maximum according to ANSI C63.4-1992, and/or EN55022:1994 /A1:1995 /A2: 1997; CISPR 22:1993/A1:1995/A2:1996.  Once the maximum reading is obtained, the antenna elevation and polarization will be varied between specified limits to maximize the readings.

During the open site measurements, the peaks are selected, the scan frequency span width is reduced to 0--1MHz, the audible modulation is monitored with a loudspeaker and the quasi-peak reading or peak is recorded at the indicated frequency and at the specified bandwidth.

## Appendix C: Test Equipment

### Test Equipment List

| Location | Equipment Name | Brand | Model | S/N | Last Cal. Date | Next Cal. Date |
| --- | --- | --- | --- | --- | --- | --- |
| Conduction | 50 Ohms Load Conduction 02 | EMCO | N/A | ISL-50ohms conduction 02 | 11/21/2001 | 11/21/2002 |
| Conduction | Coaxial Cable 1F-C2 | Harbourindustries | RG400 | 1F-C2 | 06/01/2001 | 06/01/2002 |
| Conduction | Digital Hygro-Thermometer Conduct | MicroLife | HT-2126G | ISL-Conduction02 | 12/16/2001 | 12/16/2003 |
| Conduction | EMI Receiver 03 | HP | 85460A | 3448A00209 | 11/23/2001 | 11/23/2002 |
| Conduction | LISN 01 | R&S | ESH2-Z5 | 890485/013 | 05/07/2001 | 05/07/2002 |
| Conduction | LISN 04 | EMCO | 3810/2 | 9604-1429 | 10/31/2001 | 10/31/2002 |
| Radiation | BILOG Antenna 08 | Schaffner | CBL6112B | 2756 | 04/06/2001 | 04/06/2004 |
| Radiation | Coaxial Cable Chmb 02-10M | Belden | RG-8/U | Chmb 02-10M | 01/14/2002 | 01/14/2003 |
| Radiation | Digital Hygro-Thermometer Chmb 02 | MicroLife | HT-2126G | Chmb 02 | 02/07/2002 | 02/07/2003 |
| Radiation | EMI Receiver 05 | AFJ | ER 55CR | 55390143234 | 11/07/2001 | 11/07/2002 |
| Radiation | Microwave Cable Chmb 02 3M | HUBER+SUHNER AG. | Sucoflex 103 | 42731/3 | 01/14/2002 | 01/14/2003 |
| Radiation | Spectrum Analyzer 08 | Advantest | R3132 | 111000867 | 11/21/2001 | 11/21/2002 |
| Rad. Above 1Ghz | Horn Antenna 02 | Com-Power | AH-118 | 10088 | 02/25/2002 | 02/25/2003 |
| Rad. Above 1Ghz | Horn Antenna 04 | Com-Power | AH-826 | 081-001 | 10/17/2001 | 10/17/2002 |
| Rad. Above 1Ghz | Horn Antenna 05 | Com-Power | AH-640 | 100A | 09/13/2001 | 09/13/2002 |
| Rad. Above 1Ghz | Microwave Cable RF07-4 | HUBER+SUHNER AG. | Sucoflex 103 | 42726/3 | 09/12/2001 | 09/12/2002 |
| Rad. Above 1Ghz | Preamplifier 02 | MITEQ | AFS44-00102650-40-10P-44 | 728229 | 04/17/2002 | 04/17/2003 |
| Rad. Above 1Ghz | Signal Generator 03 | Anritsu | MG3642A | 6200162550 | 02/05/2002 | 02/05/2003 |
| Rad. Above 1Ghz | Signal Generator 04 | Anritsu | MG3692A | 020311 | 02/06/2002 | 02/06/2004 |
| Rad. Above 1Ghz | Spectrum Analyzer 07 | Advantest | R3182 | 110600649 | 10/17/2001 | 10/17/2002 |

### Software for Controlling Spectrum/Receiver and Calculating Test Data

| Radiation/Conduction | Filename | Version | Issued Date |
| --- | --- | --- | --- |
| Conduction | Tile.exe | 2.0.P | 2/12/2002 |
| Radiation | Tile.exe | 2.0.P | 2/12/2002 |

## Appendix D: Layout of EUT and Support Equipment

### General Power Main Port Conducted Test Configuration

40 cm

EUT

50W

Non-Conducted Table

50W

LISN

CDN

AE

LISN

80 cm

Filter

Filter

Metal Ground Plane

Printer

Receiver

Control PC

### General Radiation Test Configuration

10 M

1-4M

Antenna and turntable distance

3 m to 10 m.

Non-Conducted

Table

To Spectrum

80 cm	To Remote

Controller

To Remote Controller

Load

Active

EUT

Printer

Spectrum

Control PC

Remote

Controller

Metal Full Soldered Ground Plane

To Turntable

To Antenna

To Antenna

## Appendix E: Description of Support Equipment

### Description of Support Equipment

- Description:	Logitech USB Mouse
Model Number:	M-u48a
Serial Number:	LZE02050204
Power Supply Type:	N/A
Power Cord:	N/A
FCC ID:	JNZ211360

- Description:	Logitech USB Mouse
Model Number:	M-u48a
Serial Number:	LZE02050204
Power Supply Type:	N/A
Power Cord:	N/A
FCC ID:	JNZ211360

- Description:	Logitech USB Mouse
Model Number:	M-u48a
Serial Number:	LZE02050204
Power Supply Type:	N/A
Power Cord:	N/A
FCC ID:	JNZ211360

- Description:	KOKA Microphone
Model Number:	DM-510
Serial Number:	N/A
Power Supply Type:	N/A
Power Cord:	N/A
FCC ID:	N/A

- Description:	Firstline Headphone
Model Number:	H1160.0
Serial Number:	N/A
Power Supply Type:	N/A
Power Cord:	N/A
FCC ID:	N/A

- Description:	Firstline Headphone
Model Number:	H1160.0
Serial Number:	N/A
Power Supply Type:	N/A
Power Cord:	N/A
FCC ID:	N/A

- Description：	Broadband Router
Model：	Router 904
Serial Number：	N/A
AC-AC Adaptor：	Linear, Power Adapter
	OEM (Model: AA-091A) 2-pin
Power Cord：	Non-shielded, Detachable

- Description:	Acer Digital Camera
Model:	DVC-VII
Serial Number:	N/A
Power Supply Type:	From PC USB Port
Power Cord:	N/A
FCC ID:	(Comply with FCC DOC)

- Description:	Logitech USB Mouse
Model Number:	M-u48a
Serial Number:	LZE02050204
Power Supply Type:	N/A
Power Cord:	N/A
FCC ID:	JNZ211360

- Description:	Firstline Headphone
Model Number:	H1160.0
Serial Number:	N/A
Power Supply Type:	N/A
Power Cord:	N/A
FCC ID:	N/A

- Description:	KOKA Microphone
Model Number:	DM-510
Serial Number:	N/A
Power Supply Type:	N/A
Power Cord:	N/A
FCC ID:	N/A

- Description:	Coson radio cassette player
Model Number:	C-2087
Serial Number:	N/A
Power Supply Type:	N/A
Power Cord:	N/A

- Description:	HP Printer (for parallel interface port)
Model Number:	C2642A
Serial Number:	TH84T1N3J3
Power Supply Type:	AC Adaptor (HP Model: C2175A)
Power Cord:	Non-shielded, Detachable
Data Cable:	Shielded, Detachable, With Metal Hood
FCC ID:	B94C2642X

- Description:	Acer Mouse
Model Number:	M-S42
Serial Number:	LZE93852045
Power Supply Type:	N/A
Power Cord:	N/A
FCC ID:	211106

- Description:	Acer Monitor
Model:	G781
Serial Number:	999007101214400445T7AA31T
Power Cord:	Non-shielded, Detachable
FCC ID:	(Comply with FCC Standards)

- Description:	Acer Keyboard
Model Number:	6511-KW
Serial Number:	K6565080970
Power Supply Type:	N/A
Power Cord:	N/A
FCC ID:	JVPKBS-WIN

- Description:	Acer USB Keyboard
Model Number:	6511-UV
Serial Number:	N/A
Power Supply Type:	N/A
Power Cord:	N/A
FCC ID:	N/A (comply with FCC DOC)

- Description:	Acer Personal Computer
Model:	VT7200
Serial No.:	N/A	
Power Supply Type:	Delta  (Model: DPS-300GB-1)3892C932
Hard Disk Drive:	Maxtor (Model:53073U6) 
Floppy Driver:	Panasonic (Model: JU256A276PC)
CD-ROM Drive:	AOpen (Model: CD-952E/AKH)
VGA Card:	LEADTEK(Model:Winfast Geforce 2 MX)
Parallel Port:	one 25-pin
Serial Port:	one 9-pin
LAN Port:	one 8-pin
Keyboard Connector:	one 6-pin
Mouse Connector:	one 6-pin
USB Connector:	two 4-pin
Game Port:	one 15-pin
Speaker Port:	one
Microphone Port:	one
Line In Port:	one
Power Cord:	Non-shielded, Detachable
Modem Card:	AMBIT(Model:T62M154)
FCC ID:	N/A (comply witch FCC DOC)

- Description:	Acer Speaker
Model Number:	A201S
Serial Number:	20101.001
Power Supply Type:	N/A
Power Cord:	Non-shielded
FCC ID:	(Comply with FCC DOC)

### Software for Controlling Support Unit

A test program which generates a complete line of continuously repeating "H" pattern is used as the software test program.  The program was executed as follows:

- Read and write to the disk drives.

- Receive audio signal from the microphone.

- Send audio signal to the headphone.

- Send audio signal to the headphone.

- Send package to the Router console port (Router).

- Send package to the Router LAN port (Router).

- Capture image from digital video camera than transfer to display (CCD)

- Send audio signal to the headphone.

- Receive audio signal from the microphone.

- Receive audio signal from walkman.

- Send H pattern to the parallel port device (Printer).

- Send H pattern to the video port device (Monitor).

- Send signal form EUT to server through Telephone port.

- Send audio signal to the speaker.

- Repeat the above steps.

|  | Filename |  |  | Issued Date |
| --- | --- | --- | --- | --- |
| Telephone |  | Hypertrm.exe | 06/08/2001 |  |
| Monitor | HH.bat |  |  | 8/20/1991 |
| Printer1 | Wordpad.exe |  |  | 11/11/1999 |
| Digital Camera | Acer Cap.exe |  |  | 8/10/1998 |
| Router LAN Port | Ping.exe |  |  | 5/5/1999 |
| Router Console Port | Commtest.exe |  |  | 8/20/2001 |

### I/O Cable Condition of EUT and Support Units

| Description | Path | Cable Length | Cable Type | Connector Type |
| --- | --- | --- | --- | --- |
| AC Power Cord | 110V (~240V) to AC Power Cord Inlet (3-pin) | 1.8M | Nonshielded, Detachable | Plastic Head |
| Telephone Line | Server to EUT RJ 11 Connector | 1.2M | Nonshielded, Detachable to PC | Plastic Head Plastic Hood |
| Telephone Line | with 600 Ωdummy load | 1.2M | Nonshielded, Detachable to PC | Plastic Head Plastic Hood |
| USB Keyboard Data Cable | USB Keyboard to PC USB port | 1.8M | Shielded, Undetachable | Metal Head |
| Keyboard Data Cable | Keyboard to PC Keyboard port | 1.8M | Shielded, Undetachable | Metal Head |
| Monitor Data Cable | Monitor to PC VGA port | 1.6M | Shielded, Un-detachable | Metal Head |
| Mouse Data Cable | Mouse to PC Mouse port | 1.8M | Shielded, Un-detachable | Metal Head |
| Printer Data Cable | Printer to PC Parallel port | 1.5M | Shielded, Detachable | Metal Head |
| Audio-in Data Cable | Walkman to PC Line In Port | 2M | Non-shielded, Detachable | Plastic Head |
| Microphone Data Cable | Microphone to PC Line In Port | 1.5M | Nonshielded, Undetachable | Plastic Head |
| Headphone Data Cable | Headphone to PC Line Out Port | 1.2M | Nonshielded, Undetachable | Plastic Head |
| USB Mouse Data Cable | USB Mouse to PC USB port | 1.8M | Shielded, Un-detachable | Metal Head |
| USB CCD Data Cable | Digital camera to PC USB port | 1.6M | Shielded, Undetachable | Metal Head |
| LAN Data Cable | EUT LAN Port to Router LAN Port. | 1.0M | Shielded, Detachable | RJ-45, with Plastic Head |

| Description |  | Path | Cable Length |  | Cable Type |  |  | Connector Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Console Cable |  | EUT COM port to Router Console Port. | 1.0M |  | Shielded, Detachable |  |  | Metal Head |
| Headphone Data Cable |  | Headphone to PC Line Out Port | 1.2M |  | Nonshielded, Undetachable |  |  | Plastic Head |
| Headphone Data Cable |  | Headphone to PC Line Out Port | 1.2M |  | Nonshielded, Undetachable |  |  | Plastic Head |
| Microphone Data Cable |  | Microphone to PC Line In Port | 1.5M |  | Nonshielded, Undetachable |  |  | Plastic Head |
| USB Mouse Data Cable |  | USB Mouse to PC USB port | 1.8M |  | Shielded, Un-detachable |  |  | Metal Head |
| USB Mouse Data Cable |  | USB Mouse to PC USB port | 1.8M |  | Shielded, Un-detachable |  |  | Metal Head |
| USB Mouse Data Cable |  | USB Mouse to PC USB port | 1.8M |  | Shielded, Un-detachable |  |  | Metal Head |
| Speaker Data Cable | Speaker to PC Line out port port |  |  | 1.5M |  | Non-shielded, Un-detachable | Plastic Head |  |
| Speaker Power Cable | Speaker to PC DC out port |  |  | 1.5M |  | Non-shielded, Un-detachable | Plastic Head |  |

## Appendix F: Description of Equipment Under Test

EUT

Description:	Personal Computer

Condition:	Pre-Production
Model:	VT7500; VT7500G
Serial Number:	N/A

Power Supply :	Switching

Delta (Model: DPS-200PB-112A) 200W or
	API (Model: API1PC47) 200W

Hard Disk Driver:	Seagate (Model: ST340016A) 40.0G

Floppy Driver:	Panasonic (Model: JU-256A047PC)

DDR:	Apacer DDR266 256MB X 2 (Model:

77.10203.540)

DVD-ROM Driver:	Pioneer (Model: DVD-117RD) 40X

CD-RW-ROM Driver:	LITE-ON (Model: LTR-40125S)40X

Modem Card:	Ambit V92 56K HSFI(Model: T62M206.00)

PS2 Connector:	two 6-pin

USB Connector:	six 4-pin

VGA Port:	one 15-pin

Parallel Port:	one 25-pin

Serial Port:	one 9-pin

Lan Port:	one 8-pin

Micophone Port:	two

Line Out Port:	two

Line In Port: 	one

Display:	CRT

Maximum Resolution:	1280 x 1024

Power cord:	Non-shielded, Detachable

Speed & CPU

Speed	CPU

133 MHz		Pentium 4 2.4GHz

100 MHz		Pentium 4 1.7, 1.8, 1.9, 2.0, 2.2, 2.4 GHz

All types of CPU with related components have been tested, only shown the worst data using model:VT7500, CPU: Pentium 4 2.4GHz/ 133MHz , SPS: Delta (Model:DPS- 200PB-112A) 200W test data in the report.

EMI Noise Source:

Crystal: 14.318MHz(X1), 25MHz(X2), 32.768MHz(X3)

Clock Generator: One Clock Generator U16

EMI Solution:

No

Different list table for model name:

| Project Name Difference | VT7500G | VT7500 |
| --- | --- | --- |
| FSB | 3.2GB/S with 400MHz FSB 4.2GB/S with 533MHz FSB | 3.2GB/S with 400MHz FSB |
| AGP SLOT | 4X Brookdale-G | Non |
| GMCH | Brookdale-G | Brookdale-GL |

## Appendix G: Uncertainty of Measurement

| Test Site: | Condcution 02 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Item | Source of Uncertainty | Probability Distribution | Total Uncertainties (dB) |  | Standard Uncertainty (dB) |  |
| 1 | Systematic Effects: (Assessment from 20 repeat observation; 1 reading on EUT) | Normal | k=2 | 0.104 | k=1 | 0.052 |
| 2 | Random Effects: (Assessment from 20 random observations; 1 reading on EUT) | Normal | k=2 | 0.330 | k=1 | 0.165 |
| 3 | Receiver Calibration | Rectangular | k=1.73 | 1.000 | k=1 | 0.577 |
| 4 | LISN Factor Calibration | Normal | k=2 | 1.200 | k=1 | 0.600 |
| 5 | Cable Loss Calibration | Normal | k=2 | 1.000 | k=1 | 0.500 |
| 6 | Combined Standard Uncertainty Uc(y) | Normal |  |  | k=1 | 0.850 |
| 7 | Total Uncertainty @95% mim. Confidence Level | Normal | k=2 | 1.701 |  |  |

Measurement Uncertainty  Calculations:

Uc (y) = square root ( u1 (y)2   + u2 (y)2 + ..........+un (y)2 )

U = 2 * Uc (y)

Note: The measurement Uncertainties mentioned above also refer to NIS 81-1994   of NAMAS : The treatment of Uncertainty in EMC Measurement.

| Test Site: | Chamber 02-10M |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Item | Source of Uncertainty | Probability Distribution | Total Uncertainties (dB) |  | Standard Uncertainty (dB) |  |
| 1 | Systematic Effects: (Assessment from 20 repeat observation; 1 reading on EUT) | Normal | k=2 | 0.134 | k=1 | 0.067 |
| 2 | Random Effects: (Assessment from 20 random observations; 1 reading on EUT) | Normal | k=2 | 0.206 | k=1 | 0.103 |
| 3 | Receiver Calibration | Rectangular | k=1.73 | 1.000 | k=1 | 0.577 |
| 4 | Antenna Factor Calibration | Normal | k=2 | 1.400 | k=1 | 0.700 |
| 5 | Cable Loss Calibration | Normal | k=2 | 1.000 | k=1 | 0.500 |
| 6 | Combined Standard Uncertainty Uc(y) | Normal |  |  | k=1 | 0.916 |
| 7 | Total Uncertainty @95% mim. Confidence Level | Normal | k=2 | 1.831 |  |  |

Measurement Uncertainty  Calculations:

Uc (y) = square root ( u1 (y)2   + u2 (y)2 + ..........+un (y)2 )

U = 2 * Uc (y)

Note: The measurement Uncertainties mentioned above also refer to NIS 81-1994   of NAMAS : The treatment of Uncertainty in EMC Measurement.

## Appendix H: Photographs of EUT Configuration Test Set Up

According to EN55022:1994 /A1:1995 /A2: 1997; CISPR 22:1993/A1:1995/A2:1996; AS/NZS 3548:1995 /A1:1997 /A2:1997:

Front View of Highest Main Power Port Conducted Emission

Front View of Highest Radiated Emission Test.

Back View of Highest Radiated Emission Test.