OPSD BIOS ENGINEERING

Intel Corporation, 5200 NE Elam Young Parkway
Hillsboro, OR  97124-6497

DATE:	December 14, 1999

TO:	OPSD Business Unit

CC:	iPG / OPSD BIOS Engineering

FROM:	iPG / OPSD BIOS Engineering

SUBJECT:	CC82010A.86A Standard BIOS P04-0006

About This Release

- CC82010A.86A.0012.P06.

### Features/Errata Fixed in This Release

#### P06-0012

- Removed Memory detection code to allow for Setup to override the CAS Latency and 1n/2n Mode parameters.

- Fixed issue that prevented the system from going into S3 when serial port loopbacks were installed.

#### P05-0011

- Added WfM 2.0 support for BIS (Boot Integrity Services).

- Changed the setup default for the Video Memory Cache Mode from "Write Combine" to "Uncached".

- Fixed WOL hang with 3COM Fast EtherLink 3C905x.

- Fixed hang whenever Novell BootWare PRO-100B v1.00 card is in the system.

- Fixed issue where ARMD-FDD devices would not work if the Floppy Controller was DISABLED and Floppy A Type was set to "Not Installed" in SETUP.

- Added error beeps if a video adapter is not detected during POST (one long beep, followed by 2 short beeps).

- Updated Memory detection code to allow for Setup to override the CAS Latency and 1n/2n Mode parameters.

- Fixed issue where FDD POST errors were getting lost.

- Fixed issue where system would delay before booting Windows NT when using a SCSI drive and not having IDE-HDD before the SCSI Adapter in the boot order.

- Fixed WHQL PC99 "IDE & Video Disable Test".

- Fixed an issue where the system would hang if the system tried to execute int19 more than one time.

- Fixed issue where going into and out of S3 state and then restarting to MSDOS mode from Win98 causes the USB Keyboard to be lost.

- Fixed issues with Type 17-20 SMBIOS memory structures.

- Updated SMBIOS Type 10 structures to correctly report the enabled/disabled status of onboard devices.

- Modified the silent boot to not display option rom messages during POST.

- Fixed an issue where the screen toggle key did not work during late POST.

- Fixed an issue where the small logo did not show up after the BIOS switched to the BIOS screen and an error message was displayed.

- Fixed an issue where the screen would be cleared during POST when unattended start was enabled.

- Increased the maximum number of PCI busses from 16 to 256.

- Fixed issue where Wake On Ring does not work with an AMR Modem is installed in Windows 98 APM Mode.

- Added support for SSDT Table to declare that the HW does not support STR when selected in Setup or if some add-in components do not support STR.

- Removed Floppy Drive from Boot Menu when Floppy Controller and Floppy Drive is not present in the system.